Make sure PCB passes DRC

Make Gerbers

Make Drill Files

Compress all files in Plots folder to ZIP and rename zip file

Create/double check BOM.xlsx of the parts JLCPCB will mount; looks like this

| Comment | Designator | Footprint | LCSC |
| --- | --- | --- | --- |
| 4.7uF | C1,C2,C10,C11 | 0805 | C1779 |
| 22uF | C5,C6,C7 | 0805 | C45783 |
| 0.1uF | C3,C4 | 0603 | C1590 |
| 680uF | C8 | SMD,D10xL10.5mm | C2980172 |
| 330uF | C9 | SMD,D10xL10.5mm | C5155335 |
| 120 | R4 | 0603 | C22787 |
| 1.5k | R2,R5 | 0603 | C22843 |
| 3.3k | R1 | 0603 | C22978 |
| 10k | R3 | 0603 | C25804 |
| 33k | R6,R7 | 0603 | C4216 |
| TCAN1057AVDRQ1 | U6 | SOIC-8 | C3235000 |
| LM2596S-5.0/NOPB | U1 | TO-263-5 | C116713 |
| LED | D1, D2 | SMD,1.6x0.8mm | C5879214 |
| BAT60AE6327 | D3,D4 | SOD-323 | C513277 |
| DRV8871DDAR | U7,U8 | SO-PowerPad-8 | C75864 |
| SMDRI127-330MT | L1 | SMD,12.3x12.3mm | C9400 |
| B530C-13-F | D5 | SMC(DO-214AB) | C110488 |
| LaunchPad | J3_J1, J2_J4 | SMD,P=2.54mm | C3294468 |
| LM2937IMP-3.3/NOPB | U2,U4 | SOT-223 | C140265 |

Generate Placement file

Edit pos file to this format, yellow are entries needing editing

| Designator | Mid X | Mid Y | Layer | Rotation |
| --- | --- | --- | --- | --- |
| C1 | 100.4570mm | -73.5660mm | top | 270 |
| C2 | 100.3300mm | -78.3920mm | top | 90 |
| C3 | 66.8020mm | -53.0860mm | top | 180 |
| C4 | 71.3870mm | -45.7200mm | top | 0 |
| C5 | 121.9200mm | -53.7870mm | top | 270 |
| C6 | 112.3950mm | -98.2320mm | top | 90 |
| C7 | 141.2240mm | -74.4830mm | top | 90 |
| C8 | 57.2770mm | -67.8270mm | top | 90 |
| C9 | 82.4230mm | -67.9360mm | top | 90 |
| C10 | 139.7610mm | -48.7680mm | top | 0 |
| C11 | 139.7610mm | -44.1960mm | top | 0 |
| D1 | 66.6750mm | -107.4420mm | top | 180 |
| D2 | 132.4610mm | -109.9820mm | top | 180 |
| D3 | 77.3430mm | -115.6970mm | top | 0 |
| D4 | 77.3430mm | -110.4900mm | top | 0 |
| D5 | 58.0390mm | -81.6320mm | top | 90 |
| J2_J4 | 92.7100mm | -54.5950mm | top | 0 |
| J3_J1 | 92.7100mm | -97.7750mm | top | 0 |
| L1 | 70.3880mm | -85.5980mm | top | 0 |
| R1 | 63.1190mm | -107.4420mm | top | 0 |
| R2 | 128.5250mm | -109.9820mm | top | 0 |
| R3 | 127.7620mm | -40.8310mm | top | 90 |
| R4 | 74.8670mm | -43.0530mm | top | 0 |
| R5 | 111.9510mm | -113.2840mm | top | 0 |
| R6 | 121.9200mm | -57.7210mm | top | 90 |
| R7 | 112.3950mm | -94.2970mm | top | 270 |
| U1 | 70.3580mm | -71.424mm | top | 270 |
| U2 | 109.7030mm | -76.1860mm | top | 180 |
| U4 | 133.2480mm | -46.4960mm | top | 0 |
| U6 | 66.5480mm | -48.0500mm | top | 0 |
| U7 | 117.0940mm | -55.5810mm | top | 0 |
| U8 | 117.0940mm | -96.4380mm | top | 180 |

Upload .zip “Add gerber file” Standard PCB/PCBA

FR-4

Qty

Industrial

Single PCB

1.6mm

Green color

White silkscreen

FR4

Leadfree

Others default

Select PCB assembly

Economic

Top side only

Default options

Add BOM file (BOM file)

Add CPL file (position file)

BOM and pos files to JLCPCB

Double check position and rotation, fix position file and reupload as needed